| ISKRATEL GROUP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| СЕРВИСНАЯ ИНФОРМАЦИЯ |  |  |  |  | Система |  |  | MPS |
| №: 5 |  | Автор: Остроумов Андрей |  |  |  | Дата: 19/09/2006 |  |  |
| Версия продукта: | Px2031,PS3013 |  | SW Release |  |  |  | Кол-во страниц: 3 |  |
| ### Teмa: Настройка компьютера для работы с системой MPS. |  |  |  |  |  |  |  |  |

1. Установка модема: Start – Settings – Control Panel – Phone and Modem Options; Modems – Add – Install new Modem – Don’t Detect My Modem ; Standard Modem Types – Communication cable between two computers COM1 – Next – Finish. 2. Конфигурация модема: Phone and Modem Options – RS232 – Properties – Communication cable between two computers – General – 19200. Advanced – Change Default Preferences Port speed - 19200 Flow control - None Data bits - 8 Parity - None Stop bits - 1 3. Конфигурирование Dial-Up: Start – Settings – Control Panel – Network and Dial-Up connections – Make New Connection – Connect directly to another computer – COM1 Имя сетевого соединения – MPS. 4. Создание записи вызывных данных: Start – Control panel – Network and Dial-Up connection – MPS User name - ppp-omt Password - ppp v Save Password Properties – Direct Connection – General – Maximum Speed – 19200; Options: v Display progress while connecting v Prompt for name and password Redial attempt - 3 Time between redial attempt - 1 minute Idle time before hanging up - never Networking – Settings – PPP settings: v Enable LCP extension v Enable software compression v Negotiate multi-link for single link connections Internet protocol (TCP/IP) – Properties – IP address – 193.7.5.102 Advanced – General – Use IP header compression DNS – v Register this connection’s addresses in DNS 5. Запись объекта в системный файл: Вписать IP-адрес объекта и его имя в файл Hosts: C:\WINNT\System32\Drivers\Ets\Hosts 193.7.5.2 MPS 6. Инсталляция JRE 1.3.1.-05i для выполнения приложений на языке Java. Запустить инсталляционный файл j2re-1_3-1_05-windows-i586-.exe После инсталляции перезагрузка не требуется. Настройка: Start – programs – Java plug-in 1.3.1_05 Basic - v Enable Java plug-in v Show Java Console Advanced – JRE 1/3/1_05 in C:\Program Files\JavaSoft\JRE\1.3.1_05 Browser – v Microsoft internet Explorer Proxies – v Use browser settings Cache – Clear JAR Cache 7. Инсталляция Java Web Start 1.0.1. Запустить webstart_1_0_1.exe Accept – Next – OK. Перезагрузка компьютера. 8. Настройка Internet Explorer. Start – Settings – Control panel – Internet Options. General – Temporary Internet files – Settings – Never 9. Установление соединения: Start – Settings – Network and Dial-Up Connections (MPS) ppp-omt ppp Connect. Java Plug-in Security Warning – Grant always File Download – Open this file from its current location Security Warning – Start.