| НАЗВАНИЕ: Определение максимального количества файлов на диске станции |  |  |  |  |
| --- | --- | --- | --- | --- |
| ДЕЙСТВИТЕЛЬНО ДЛЯ ПРОДУКТА | Все продукты |  |  |  |
| КРАТКОЕ ОПИСАНИЕ: Определение максимального количества файлов на диске при его инициализации. Максимальное количество файлов на диске программы инсталляции и загрузки, предлагаемое до сих пор, составляло 1000. Новое значение по умолчанию – 1000 файлов на дисках, емкость которых не превышает 100 МБ, или 10000 на дисках с большой емкостью (относится к версии MMWB0A08 и выше). Предлагаемое значение по умолчанию соответствует модулям MLC в функции модуля доступа AN. Если для модулей MLC в функции SAN используются диски небольшой емкости (минимальная допустимая емкость диска составляет 80 МБ), предлагается определение количества файлов на диске - 10000, но ни в коем случае меньше чем 2000. Параметр количества файлов на диске устанавливается через меню, которое открывается нажатием кнопки в течение соответствующего интервала времени (startup delay) после запуска или сброса системы – ниже дается пример диска, емкость которого меньше 100 МБ. Значение по умолчанию для дисков с большой емкостью – 10000 файлов. To change STARTUP MODE or system settings, press any key within 10 seconds 1 - Run-time installation 2 - Development installations 3 - System console 4 - Disk configuration 5 - Network configuration 6 - Other configuration 7 - Read configuration from file 8 - Advanced configuration Please, select the option you want: [1] 4 1 - Boot dir 2 - Disk verification mode 3 - Format & initialize disk 4 - Initialize disk 5 - Select BOOT media X - Return to the main menu Please, select the option you want: [] 4 Do you really want to initialize disk? [N] y Updating parameter storage. This may take a while...Done Last loading ...... Ok System time ....... 11:32:24 03/01/2050 Disk .............. Transaction control records at 0x00010000 (DRAM) ... Last transaction completed successfully. ... DISK 0 SunDisk SD25B-32 MT311850216 vcb 1.45 Size 30 Full disk capacity is 30 Mb (62560 blocks). Do you want to reduce volume size? [N] Enter max number of files on the file system: [1000] Max number of files is 2000. I'M INITIALIZING DISK DEVICE NOW ... Done Writting system parameters to file ... Done Copyright (c) Integrated Systems, Inc., 1992. Welcome to pSOSystem... pSH+> |  |  |  |  |